FINGER PRINT SCANNER:

Computerized fingerprint scanners have been a mainstay of spy thrillers for decades, but up until recently, they were pretty exotic technology in the real world. In the past few years, however, scanners have started popping up all over the place -- in police stations, high-security buildings and even on PC keyboards. You can pick up a personal USB fingerprint scanner for less than $100, and just like that, your computer's guarded by high-tech biometrics. Instead of, or in addition to, a password, you need your distinctive print to gain access.

In this article, we'll examine the secrets behind this exciting development in law enforcement and identity security. We'll also see how fingerprint scanner security systems stack up to conventional password and identity card systems, and find out how they can fail.

# Fingerprint Basics

Fingerprints are one of those bizarre twists of nature. Human beings happen to have built-in, easily accessible identity cards. You have a unique design, which represents you alone, literally at your fingertips. How did this happen?

People have tiny ridges of skin on their fingers because this particular adaptation was extremely advantageous to the ancestors of the human species. The pattern of ridges and "valleys" on fingers make it easier for the hands to grip things, in the same way a rubber tread pattern helps a tire grip the road.

The other function of fingerprints is a total coincidence. Like everything in the human body, these ridges form through a combination of genetic and environmental factors. The genetic code in DNA gives general orders on the way skin should form in a developing fetus, but the specific way it forms is a result of random events. The exact position of the fetus in the womb at a particular moment and the exact composition and density of surrounding amniotic fluid decides how every individual ridge will form.

So, in addition to the countless things that go into deciding your genetic make-up in the first place, there are innumerable environmental factors influencing the formation of the fingers. Just like the weather conditions that form clouds or the coastline of a beach, the entire development process is so chaotic that, in the entire course of human history, there is virtually no chance of the same exact pattern forming twice.

Consequently, fingerprints are a unique marker for a person, even an identical twin. And while two prints may look basically the same at a glance, a trained investigator or an advanced piece of software can pick out clear, defined differences.

This is the basic idea of fingerprint analysis, in both crime investigation and security. A fingerprint scanner's job is to take the place of a human analyst by collecting a print sample and comparing it to other samples on record.

A fingerprint scanner system has two basic jobs  it needs to get an image of your finger, and it needs to determine whether the pattern of ridges and valleys in this image matches the pattern of ridges and valleys in pre-scanned images.

There are a number of different ways to get an image of somebody's finger. The most common methods today are optical scanning and capacitance scanning. Both types come up with the same sort of image, but they go about it in completely different ways.

The heart of an optical scanner is a charge coupled device (CCD), the same light sensor system used in digital cameras and camcorders. A CCD is simply an array of light-sensitive diodes called photosites, which generate an electrical signal in response to light photons. Each photosite records a pixel, a tiny dot representing the light that hit that spot. Collectively, the light and dark pixels form an image of the scanned scene (a finger, for example). Typically, an analog-to-digital converter in the scanner system processes the analog electrical signal to generate a digital representation of this image. See How Digital Cameras Work for details on CCDs and digital conversion.

The scanning process starts when you place your finger on a glass plate, and a CCD camera takes a picture. The scanner has its own light source, typically an array of light-emitting diodes, to illuminate the ridges of the finger. The CCD system actually generates an inverted image of the finger, with darker areas representing more reflected light (the ridges of the finger) and lighter areas representing less reflected light (the valleys between the ridges).

Finger print identification:

Fingerprint identification is one of the most important biometric technologies which have drawn a substantial amount of attention recently. A fingerprint is the pattern of ridges and valleys (also called furrows in the fingerprint literature) on the surface of a fingertip. Each individual has unique fingerprints. The uniqueness of a fingerprint is exclusively determined by the local ridge characteristics and their relationships. A total of 150 different local ridge characteristics (islands, short ridges, enclosure, etc.) have been identified. These local ridge characteristics are not evenly distributed. Most of them depend heavily on the impression conditions and quality of fingerprints and are rarely observed in fingerprints. The two most prominent local ridge characteristics, called minutiae, are

1. Ridge ending.

2. Ridge bifurcation.

A ridge ending is defined as the point where a ridge ends abruptly. A ridge bifurcation is defined as the point where a ridge forks or diverges into branch ridges. A good quality fingerprint typically contains about 40 to 100 minutiae. Examples of minutiae are shown in the following Figure.

Examples of minutiae. (a) A minutiae can be characterized by its

position and its orientation. (b) Minutiae overlaid on a fingerprint Image.

Automatic fingerprint matching depends on the comparison of these local ridge characteristics and their relationships to make a personal identification. A critical step in fingerprint matching is to automatically and reliably extract minutiae from the input fingerprint images, which is a difficult task. The performance of a minutiae extraction algorithm relies heavily on the quality of the input fingerprint images.

In an ideal fingerprint image, ridges and valleys alternate and flow in a locally constant direction and minutiae are anomalies of ridges, i.e., ridge endings and ridge bifurcations. In such situations, the ridges can be easily detected and minutiae can be precisely located from the thinned ridges. Fig. 1 shows an example of good quality live scan fingerprint image. However, in practice, due to variations in impression conditions, ridge configuration, skin conditions (aberrant formations of epidermal ridges of fingerprints, postnatal marks, occupational marks), acquisition devices, and non cooperative attitude of subjects, etc., a significant percentage of acquired fingerprint images (approximately 10 percent) is of poor quality. The ridge structures in poor-quality fingerprint images are not always well-defined and, hence, they cannot be correctly detected. This leads to following problems:

1. a significant number of spurious minutiae may be created.

2. A large percent of genuine minutiae may be ignored.

3. Large errors in their localization (position and orientation) may be introduced.

In order to ensure that the performance of the minutiae extraction algorithm will be robust with respect to the quality of input fingerprint images, an enhancement algorithm which can improve the clarity of the ridge structures is necessary. A fingerprint expert is often able to correctly identify the minutiae by using various visual clues such as local ridge orientation, ridge continuity, ridge tendency, etc., as long as the ridge and valley structures are not corrupted completely. It is possible to develop an enhancement algorithm that exploits these visual clues to improve the clarity of ridge structures in corrupted fingerprint images.

Before comparing the print to stored data, the scanner processor makes sure the CCD has captured a clear image. It checks the average pixel darkness, or the overall values in a small sample, and rejects the scan if the overall image is too dark or too light. If the image is rejected, the scanner adjusts the exposure time to let in more or less light, and then tries the scan again.

If the darkness level is adequate, the scanner system goes on to check the image definition (how sharp the fingerprint scan is). The processor looks at several straight lines moving horizontally and vertically across the image. If the fingerprint image has good definition, a line running perpendicular to the ridges will be made up of alternating sections of very dark pixels and very light pixels.

If the processor finds that the image is crisp and properly exposed, it proceeds to comparing the captured fingerprint with fingerprints on file. We'll look at this process in a minute, but first we'll examine the other major scanning technology, the capacitive scanner.

Polling unit:

Polling unit improves the voting process for both electors and administrative authorities at all levels by providing an innovative infrastructure for supporting remote voting based on leading-edge technology.

Using E-Poll, constituents can vote wherever they may be on Election Day. With the new model, the concept of a district associated with an electoral register is replaced by a network allowing delocalization of the booths. E-Poll aims to bridge the gap between the availability of leading-edge technology and the practical possibility of exploiting it, by dealing with the following issues:

* Need to adapt legislation

* Need for full confidence in the voting process on the part of voters and the administrative authorities

* Guarantee of security and reliability

* Preservation of current roles in the election process.