GPS

| Definition | Specification |
| --- | --- |
| Receiver characteristic | 16 Parallel GPS channel, L1 1575.42 Ghz, C/A Code ( 1.023 Mhz )chip rate |
| RF Receiver | 16 channel receiver with on-chip reference oscillator & built-in LNA |
| Base board processor | 32-Bit Processor |
| Flash Memory | 4Mbit or 8 Mbit |
| Performance TTFF |  |
| Cold Start | 45 Seconds @-135 dBm |
| Warm Start | 34 Seconds @-140 dBm |
| Hot Start | 5 Seconds @-140 dBm |
| TT Requisition | <3 Seconds @-147 dBm |
| Sensitivity |  |
| Acquisition | -145 dBm |
| Tracking | -150 dBm |
| Accuracy |  |
| CEP50 | 1.2m |
| CEP95 | 3.0m |
| Maximum Acceleration | 3g |
| Maximum Speed | 515m/s |
| Maximum Altitude | 18Km |
| Interface |  |
| UART | 1 UART with data NMEA 0183 data format |
| GPIOs | 3vGPIOs for GPS |
| Power consumption |  |
| Full active | 25mA @ 3.3V without antenna |
| Average | 0.5 mA @ 3.3V for 1 Hz Update rate |
| Current Consumption | 25 mA @ Normal Mode |
|  | 18 mA @ Standby Mode |
|  | 4 mA @ doze – 1 mode |
|  | 100 uA @ doze – 2 mode |
|  | 10 uA @ Sleep mode |
| Power Supplies |  |
| Power Supply Voltage | 3.1 v to 3.6 V |
| Back up Voltage | 1.2v to 1.8v |
| Absolute Maximum Rating |  |
| Operating Temperature | -20 to 70 º C ( Optional for -40 to -85º C ) |
| Storage Temperature | -45 to 90º C |
| Max Current into I/O pins | + or - 50 mA |

GSM Modem Spec

Tri-Band GSM/GPRS 900/1800/1900MHz or Quad-Band GSM.GPRS 850/900/1800/1900MHz
GPRS multi-slot class 10/8
GPRS mobile station class B
Compliant to GSM phase 2/2+
     Class 4 (2W @ 900MHz)
     Class 1 (1W @ 1800/1900MHz)
Dimensions: 40mm*33mm*2.85mm
Weight: 8g
Control via AT commands (GSM 07.07, 07.05 and enhanced AT Commands)
SIM application toolkit
Supply voltage range 3.4V - 4.5V
Low power consumption
Normal operation temperature:
     -30°C to +70°C
Restricted operation temperature: 
     -35°C to +80°C
Storage temperature:
     -40°C to +85°C