Single-Chip Voice Recording & Playback Device

Features

• Single-chip, high-quality voice recording & playback solution

- No external ICs required

- Minimum external components

• Non-volatile Flash memory technology

- No battery backup required

• User-Selectable messaging options

- Random access of multiple fixed-duration messages

- Sequential access of multiple variable-duration messages

• User-friendly, easy-to-use operation

- Programming & development systems not required

- Level-activated recording & edge-activated play back switches

• Low power consumption

- Operating current: 25 mA typical

- Standby current: 1 uA typical

- Automatic power-down

• Chip Enable pin for simple message expansion

PIN DIAGRAM

The APR9600 device offers true single-chip voice recording, non-volatile storage, and playback capability for 40 to 60 seconds. The device supports both random and sequential access of multiple messages. Sample rates are user-selectable, allowing designers to customize their design for unique quality and storage time needs. Integrated output amplifier, microphone amplifier, and AGC circuits greatly

Simplify system design. the device is ideal for use in portable voice recorders, toys, and many other consumer and industrial applications. APLUS integrated achieves these high levels of storage capability by using its proprietary analog/multilevel storage technology implemented in an advanced Flash non-volatile memory process, where each memory cell can store 256 voltage levels. This technology enables the APR9600 device to reproduce voice signals in their natural form. It eliminates the need for encoding and compression, which often introduce distortion.

Functional Description

The APR96 0 0 block diagram is included in order to give understanding of the APR9600 i in ternal architecture. At the left hand side of the diagram are the analog inputs. A differential microphone amplifier, including integrated AGC, is included on-chip for applications requiring its use. The amplified microphone signal is fed into the device by connecting the Ana_Out pin to the Ana_In pin through an external DC blocking capacitor. Recording can be fed directly into the Ana_In pin through a DC blocking capacitor, however, the connection between Ana_In and Ana_Out is still required for playback. The next block encountered by the input signal is the internal anti-aliasing filter. The filter automatically adjusts its response according to the sampling frequency selected so Shannon’s Sampling Theorem is satisfied. After anti-aliasing filtering is accomplished the signal is ready to be clocked into the memory array. This storage is accomplished through a combination of the Sample and Hold circuit and the Analog Write/Read circuit. These circuits are clocked by either the Internal Oscillator or an external clock source. When playback is desired the previously stored recording is retrieved from memory, low pass filtered, and amplified as shown on the right hand side of the diagram. The signal can be heard by connecting a speaker to the SP+ and SP- pins. Chip-wide

management is accomplished through the device control block shown in the upper right hand corner. Message management is controlled through the message control block represented in the lower center of the block diagram. More detail on actual device application can be found in the Sample Applications section. More detail on sampling control can be found in the Sample Rate and Voice Quality section. More

detail on message management and device control can be found in the Message Management section

Message Management

Message Management General Description

Playback and record operations are managed by on chip circuitry.

There are several available messaging modes

depending upon desired operation. These message modes

determine message management style, message length, and

external parts count. Therefore, the designer must select the

appropriate operating mode before beginning the design.

Operating modes do not affect voice quality; for information

on factors affecting quality refer to the Sampling Rate & Voice

Quality section.

The device supports three message management modes

(defined by the MSEL1, MSEL2 and /M8_Option pins shown

in Figures 1 and 2):

• Random access mode with 2, 4, or 8 fixed-duration messages

• Tape mode, with multiple variable-duration messages, provides

two options:

- Auto rewind

- Normal

Modes cannot be mixed. Switching of modes after the device

has recorded an initial message is not recommended. If

modes are switched after an initial recording has been made

some unpredictable message fragments from the previous

mode may remain present, and be audible on playback, in

the new mode. These fragments will disappear after a record

operation in the newly selected mode. Table 1 defines the

decoding necessary to choose the desired mode.

An important feature of the APR9600 message management

capabilities is the ability to audibly prompt the user to

changes in the device’s status through the use of “beeps”

superimposed on the device’s output. This feature is enabled

by asserting a logic high level on the BE pin.